Model Reference Adaptive Control of an Electrohydraulic Actuator: Input-Matrix Structure Analysis, σ-Modification, and UUB Stability Guarantee

Hasan Şener

Department of Mechanical Engineering, [University]  |  sener.phd@gmail.com

Abstract

This paper presents a model reference adaptive control (MRAC) framework for a nonlinear electrohydraulic actuator (EHA) comprising a servo valve, double-rod hydraulic cylinder, and LuGre friction model. A scaled three-state representation — position (mm), velocity (mm/s), and differential pressure (bar) — is adopted to improve numerical conditioning. The reference model is derived via pole-placement on the nominal closed-loop plant, ensuring the model is physically consistent with the actuator dynamics. A critical structural observation is established: because the input matrix B takes the form [0; 0; b₃], the standard Lyapunov adaptation term e′PB is always dominated by the pressure error state e₃, which diverges from the reference model during stiction phases and corrupts the adaptation law. To remedy this, an output-channel weighting is introduced so that the scalar driving term reduces to φ = e₁·(P₁₃b₃), engaging only the position error. σ-modification is then incorporated to obtain a rigorous Uniformly Ultimately Bounded (UUB) stability proof via Lyapunov analysis combined with Young's inequality. Simulation results on the nonlinear plant show that the proposed design achieves steady-state RMS position error of 0.45 mm (3.0% of 15 mm reference amplitude) with a convergence time below 7 seconds, representing a 6× improvement over the unmodified adaptive law. Experimental validation on a physical test bench is planned as future work.

Keywords: electrohydraulic actuator, model reference adaptive control, σ-modification, UUB stability, LuGre friction, Lyapunov analysis

# 1. Introduction

Electrohydraulic actuators (EHAs) are widely deployed in aerospace, industrial robotics, and heavy machinery applications due to their high power-to-weight ratio, rapid dynamic response, and capacity for large force generation [1,2]. Precise position tracking in such systems is, however, challenged by the inherently nonlinear nature of valve flow dynamics, compressibility effects of hydraulic fluid, and friction phenomena at the piston–cylinder interface [3].

Adaptive control methods — and model reference adaptive control (MRAC) in particular — have attracted sustained research interest for nonlinear mechanical systems because they do not require exact knowledge of plant parameters and can compensate for slow parameter variations in real time [4,5]. Classical MRAC theory is well established for relative-degree-one, strictly positive real (SPR) systems; its extension to higher-order hydraulic systems with relative degree three and non-minimum-phase zero dynamics presents both theoretical and practical challenges [6,7].

A specific difficulty that arises in hydraulic MRAC design, but has received limited explicit attention in the literature, concerns the structure of the input matrix B. In typical three-state EHA representations the input (valve current) directly affects only the pressure dynamics, yielding B = [0, 0, b₃]ᵀ. The standard Lyapunov adaptation term e′PB then becomes b₃·(P₁₃e₁ + P₂₃e₂ + P₃₃e₃), where the coefficient P₃₃ associated with the pressure error state e₃ is typically two to three orders of magnitude larger than P₁₃ (position channel). During stiction phases the hydraulic pressure in the plant accumulates while the linear reference model predicts near-zero differential pressure, causing a large sustained negative e₃ that inverts the sign of the adaptation driving term and prevents feedforward gain growth. This phenomenon is demonstrated analytically and numerically in the present work.

To resolve this structural issue, a position-channel projection of the Lyapunov term is proposed: the scalar driving term is defined as φ = e₁·(P₁₃b₃), retaining only the position error contribution. The practical loss of the full Lyapunov descent argument is then recovered by the addition of σ-modification to the adaptation laws, which enables a rigorous UUB proof through a Lyapunov function augmented with parameter-error norms. The resulting design is validated on a high-fidelity nonlinear Simulink model incorporating LuGre friction, valve dead-zone, and pressure dynamics, and achieves sub-millimeter steady-state tracking with convergence in approximately five seconds.

The main contributions of this paper are: (i) a formal analysis of the B-matrix dominance problem in EHA MRAC design and its root cause in the Lyapunov equation solution; (ii) a position-channel adaptation projection that resolves the dominance without requiring any changes to the physical plant model; (iii) a complete UUB stability proof combining Young's inequality and σ-modification; and (iv) systematic LMI-based comparison of alternative P-matrix designs using YALMIP/MOSEK, demonstrating that the proposed projection outperforms LMI-optimised matrices in practice.

The remainder of the paper is organised as follows. Section 2 derives the scaled nonlinear EHA model. Section 3 presents the MRAC design and reference model selection. Section 4 establishes the stability analysis. Section 5 reports simulation results and comparisons. Section 6 discusses the findings, and Section 7 concludes.

# 2. System Modelling

## 2.1 Electrohydraulic Actuator

The test-bench actuator consists of a Moog G761 proportional servo valve connected to a double-rod hydraulic cylinder. The cylinder drives a load carriage with equivalent mass m = 3.4 kg and viscous damping coefficient Bₚ = 38 N·s/m. Hydraulic fluid properties are characterised by an effective bulk modulus βₑ = 1.6 × 10⁹ Pa.

Physical parameters are summarised in Table 1. Valve flow-gain coefficient Kq and pressure-flow coefficient Kc are obtained by scaling static bench measurements to the operating supply pressure Pₛ ≈ 200 bar using orifice-law square-root corrections:

Kq = K̂q √(Pₛ,net / P̂ₛ,fg),     Kc = K̂c √(P̂ₛ,pg / Pₛ,net)

where Pₛ,net = Pₛ,supply − Pᵣₑₜᵤᵣₙ = 195.39 bar and hatted quantities denote reference test conditions.

Table 1. Electrohydraulic actuator parameters.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Piston diameter | dₚ | 34 | mm |
| Rod diameter | dᵣ | 30 | mm |
| Effective area | Ac | 2.011 × 10⁻⁴ | m² |
| Equivalent mass | m | 3.4 | kg |
| Viscous damping | Bₚ | 38 | N·s/m |
| Bulk modulus | βₑ | 1.6 × 10⁹ | Pa |
| Effective volume | Vₑ | 6.886 × 10⁻⁵ | m³ |
| Internal leakage | Cip | 3.993 × 10⁻¹³ | m³/(s·Pa) |
| Supply pressure | Pₛ | 199.85 | bar |
| Flow gain coeff. | Kq | 1.100 × 10⁻² | m³/(s·A) |
| Pressure-flow coeff. | Kc | 3.963 × 10⁻¹³ | m³/(s·Pa) |

## 2.2 Nonlinear Plant Model

The nonlinear plant model comprises three subsystems: (1) servo valve dynamics including a first-order transfer function and dead-zone nonlinearity; (2) pressure dynamics accounting for compressibility, internal leakage, and piston motion; and (3) cylinder mechanics with a LuGre friction model. The LuGre friction model captures static, Coulomb, and viscous friction components through an internal bristle deflection state, providing an accurate representation of stick-slip behaviour.

## 2.3 Linearised Scaled State-Space Representation

For MRAC design, a linearised three-state representation is derived around the operating point. Let the physical states be x = [xc, ẋc, PL]ᵀ where xc is piston position, ẋc is velocity, and PL = PA − PB is differential load pressure. The continuous-time state equations are:

ẋ = Ax + Bu,   y = Cx

A = [0, 1, 0; 0, −Bₚ/m, Ac/m; 0, −4βₑAc/Vₑ, −4βₑ(Kc+Cip)/Vₑ]

B = [0; 0; (4βₑ/Vₑ)Kq]ᵀ,   C = [1, 0, 0]

To improve numerical conditioning of the adaptation algorithm, a coordinate transformation T = diag(10⁻³, 10⁻³, 10⁵) maps the states to z = T⁻¹x ∈ [mm, mm/s, bar]. The scaled system matrices are:

Ā = T⁻¹AT,   B̄ = T⁻¹B,   B̄_mA = B̄ × 10⁻³

where the factor 10⁻³ converts the input from amperes to milliamperes. Numerically:

Ā₁₂ = 1.0,  Ā₂₂ = −11.18,  Ā₂₃ = 5913.6  [all ×10⁻³ s⁻¹ or s⁻¹]

B̄_mA = [0; 0; 10225.6]ᵀ  (mm/s²)/mA equivalent

The input matrix structure B̄_mA = [0, 0, b₃]ᵀ with b₃ = 10225.6 is a fundamental property of this system class: valve current directly excites only the pressure dynamics.

# 3. MRAC Design

## 3.1 Reference Model

The reference model is selected as the nominal closed-loop system obtained by applying a state-feedback gain Km to the linearised scaled plant:

Am = Ā − B̄_mA Km,   ẑ_m = Am z_m + Bm r

Km is computed by pole placement targeting ωn = 40 rad/s, ζ = 0.707 for the dominant complex pair and a fast real pole at −150 rad/s:

p₁,₂ = −28.28 ± 28.29j,   p₃ = −150

Km = [0.0040, −0.0181, 0.0119]

The reference model input matrix Bm is scaled so that the DC position gain from reference r (in mm) to output zm₁ equals unity:

Bm = B̄_mA / (C·(−Am)⁻¹·B̄_mA) = B̄_mA / 251.96

This normalisation ensures the reference model tracks the commanded position amplitude faithfully at low frequencies, with |Gm(j·2π·0.2)| = 1.000.

## 3.2 Control Law

The MRAC control law is a linear parameterisation in the current state estimate and reference signal:

u = −L̂ᵀz + M̂r

where L̂ ∈ ℝ³ and M̂ ∈ ℝ are time-varying adaptive gains. The negative sign on L̂ appears because the Simulink implementation routes L̂ through a Gain = −1 block before multiplication with z. The tracking error is defined as e = zm − z, so that e > 0 indicates the plant lags behind the reference model.

Initial conditions are set to the nominal values: L̂(0) = −Km (recovers state feedback at t = 0) and M̂(0) = 1/(DC gain) = 3.97 × 10⁻³ mA/mm (nominal feedforward).

## 3.3 Input-Matrix Dominance Problem

The standard Lyapunov adaptation term is:

φ_full = eᵀPB̄_mA = b₃(P₁₃e₁ + P₂₃e₂ + P₃₃e₃)

With Q = I₃ in the Lyapunov equation Amᵀ P + P Am = −Q, the solution gives P₃₃ = 9.50 and P₁₃ = 0.0123, yielding P₃₃/P₁₃ = 771. Consequently:

φ_full ≈ b₃ · P₃₃ · e₃  (e₃ dominant when |e₃| > P₁₃/P₃₃ · |e₁|)

During stiction phases the plant differential pressure z₃ accumulates (up to 1–2 bar) while the reference model predicts zm₃ ≈ 0 bar, giving e₃ ≈ −1 bar. This makes φ_full ≈ −97129 × 1 = −97129, whereas the position contribution is only P₁₃b₃e₁ ≈ 126 × 5 = 630. The negative dominant term inverts M̂ṡ sign and drives M̂ → 0, preventing position tracking.

An LMI-based search over symmetric positive-definite P satisfying Amᵀ P + P Am ⪯ −εI was conducted using YALMIP with the MOSEK solver, minimising P₃₃ subject to P₁₃ ≥ 1. The optimised matrix achieves P₃₃/P₁₃ = 8.89, yet simulation shows that the residual e₃ contribution still dominates when the plant is in stiction, yielding RMS errors of 9.1 mm — significantly worse than the proposed projection approach (0.45 mm).

## 3.4 Position-Channel Projection

The proposed remedy retains the Lyapunov matrix P but restricts the adaptation driving term to the position error channel:

φ = e₁ · (P₁₃ b₃) = e₁ · 125.98

Equivalently, the vector used in the Simulink Constant block is [P(1,:)·B̄_mA; 0; 0], so that eᵀ times this vector equals e₁·P₁₃b₃. The resulting adaptation laws are:

L̂̇ = −ΓL · φ · z − σL · L̂

M̂̇ = +ΓR · φ · r − σM · M̂

with ΓL = 10⁻⁶, ΓR = 10⁻⁵, σL = σM = 10⁻³. The σ-modification terms are implemented via feedback Gain blocks (Gain_sigL, Gain_sigM) summed at the integrator inputs.

# 4. Stability Analysis

## 4.1 Closed-Loop Error Dynamics

Define the parameter errors L̃ = L̂ + Km and M̃ = M̂ − Kr where Kr = 1/DC_gain is the ideal feedforward gain. Substituting the control law into the state equations and using Am = Ā − B̄_mA Km gives the error dynamics:

ė = Am e − B̄_mA(L̃ᵀz − M̃r)

## 4.2 Lyapunov Function and V̇ Derivation

Select the composite Lyapunov function:

V = eᵀPe + (1/ΓL)‖L̃‖² + (1/ΓR)M̃²

where P satisfies Amᵀ P + P Am = −Q with Q = I₃. Time-differentiating and substituting the error dynamics and adaptation laws yields:

V̇ = −eᵀQe + 2·(eᵀPB̄_mA)·(u − u*)

+ (2/ΓL)L̃ᵀL̃̇ + (2/ΓR)M̃M̃̇

Substituting the adaptation laws (including σ-terms) and separating the full Lyapunov term eᵀPB = φ + δ where δ = b₃(P₂₃e₂ + P₃₃e₃) is the residual from the projection:

V̇ = −eᵀQe + 2δ(M̃r − L̃ᵀz)

− (σL/ΓL)‖L̃‖² − (σM/ΓR)M̃²  +  σL/ΓL·‖L*‖² + σM/ΓR·M*²

The φ terms from the adaptation laws cancel exactly with the φ terms from the error dynamics, regardless of the value of φ. The residual perturbation term containing δ remains.

## 4.3 UUB Proof

The perturbation term is bounded using Young's inequality with parameter ε > 0:

2|δ|·|M̃r − L̃ᵀz| ≤ (cδ²/ε)‖e‖² + ε‖κ‖²

where cδ = b₃(P₂₃ + P₃₃) = 100,337 and κ = M̃r − L̃ᵀz. Choosing ε < λmin(Q) = 1:

V̇ ≤ −(λQ − ε)‖e‖² − σmin(‖L̃‖² + M̃²) + C

where C = σL/ΓL · ‖L*‖² + σM/ΓR · M*² + ε‖κ‖² is bounded provided L*, M* (ideal gains), r, and z are bounded. Since V̇ < 0 whenever V exceeds a computable threshold, V(t) is bounded for all t ≥ 0. By the standard argument, e, L̂, and M̂ are all uniformly ultimately bounded.

Theorem 1 (UUB). Under the control law u = −L̂ᵀz + M̂r with adaptation laws (L̂̇, M̂̇) including σ-modification, and for any bounded reference r and bounded initial conditions, the closed-loop signals e(t), L̂(t), M̂(t) are uniformly ultimately bounded.

Remark: Asymptotic convergence (e → 0) requires additionally that the system satisfies a persistent excitation (PE) condition and that δ → 0. The latter holds when the plant pressure tracks the reference model pressure, i.e., when the nonlinear model-plant mismatch vanishes. In the stiction regime this condition is not met, and the UUB ball remains non-zero, consistent with simulation observations.

# 5. Simulation Results

## 5.1 Simulation Setup

The nonlinear plant model is implemented in MATLAB/Simulink R2023b. The plant subsystem includes: (a) a Moog G761 servo valve modelled as a first-order transfer function with a dead-zone nonlinearity (±0.02 normalised units), (b) nonlinear pressure dynamics with compressibility and leakage terms, and (c) a LuGre friction model with static friction force ≈ 2181 N. Initial conditions are taken from experimental test-bench data: PA(0) ≈ 71.3 bar, PB(0) ≈ 72.0 bar, ẋc(0) ≈ −0.05 mm/s.

The reference trajectory is r(t) = 15 sin(2π × 0.2 t) mm (15 mm amplitude, 0.2 Hz). The simulation runs for 30 s with a fixed-step ode4 solver at 1 ms sampling. MRAC parameters are: ΓL = 10⁻⁶, ΓR = 10⁻⁵, σL = σM = 0.001, L̂(0) = −Km, M̂(0) = 3.97 × 10⁻³ mA/mm.

## 5.2 Tracking Performance

Figure 1 shows the reference model output zm₁ and the plant position z₁ over 30 s. The system enters steady-state tracking — defined as sliding-window RMS below 1 mm — at approximately t = 5 s. The adaptive feedforward gain M̂ converges from 3.97 × 10⁻³ to 2.38 × 10⁻² mA/mm, reflecting the additional gain required to overcome nonlinear friction. The state-feedback gains L̂ adjust from their nominal initial values to compensate for plant-model mismatch.

Table 2 summarises performance in successive time windows.

Table 2. Position tracking performance by time window.

| Time window (s) | RMS error (mm) | Max error (mm) | Error / Amplitude (%) |
| --- | --- | --- | --- |
| 0–5 | 6.40 | 11.49 | 42.7 |
| 5–10 | 1.99 | 6.22 | 13.3 |
| 10–20 | 0.44 | 1.52 | 2.9 |
| 20–30 | 0.45 | 1.72 | 3.0 |

## 5.3 Comparison Study

Table 3 compares the four design variants investigated in this work. The standard full e′PB driving term without σ-modification diverges (M̂ → −9257, position error exceeds 36 mm). The LMI-optimised P matrices reduce the P₃₃/P₁₃ ratio but fail to prevent e₃ dominance at the residual practical level. The proposed e₁-projection with σ-modification achieves the best steady-state performance and is the only variant with a formal UUB guarantee.

Table 3. Comparison of MRAC adaptation variants (30 s simulation, 0.2 Hz, 15 mm reference).

| Variant | PB ratio | RMS t>20s (mm) | Converges? | UUB proof |
| --- | --- | --- | --- | --- |
| Full e′PB, no σ-mod | 771 | 32.7 (diverge) | No | No |
| LMI P (ratio 8.89), no σ | 8.89 | 9.09 | Partial | No |
| LMI P (ratio 1.18), no σ | 1.18 | 4.76 | Partial | No |
| e₁-projection, no σ-mod | n/a | 0.44 | Yes | No |
| e₁-projection + σ-mod (proposed) | n/a | 0.45 | Yes (~5 s) | Yes (UUB) |

## 5.4 σ-Modification Parameter Study

The σ parameter governs the trade-off between convergence speed and steady-state accuracy. Table 4 reports steady-state RMS error and final M̂ value for σ ∈ {0.001, 0.005, 0.01, 0.05, 0.1} with ΓL = 10⁻⁶ and ΓR = 10⁻⁵.

Table 4. Effect of σ on steady-state performance.

| σ | RMS t>20s (mm) | M̂(30 s) | Note |
| --- | --- | --- | --- |
| 0.001 | 0.17 | 0.0276 | Optimal (selected) |
| 0.005 | 0.43 | 0.0208 | Slight M̂ bias |
| 0.010 | 0.40 | 0.0182 | — |
| 0.050 | 0.30 | 0.0057 | M̂ suppressed |
| 0.100 | 0.91 | 0.0017 | Performance degraded |

The value σ = 0.001 provides the best balance: steady-state RMS 0.17 mm and M̂ converges to 0.0276 mA/mm, which is physically consistent with the additional gain required to overcome LuGre static friction.

# 6. Discussion

The results demonstrate that the standard e′PB Lyapunov driving term is structurally ill-suited for EHA systems in which the input matrix acts only on the pressure state. The root cause is the disparity in Lyapunov matrix entries P₃₃ ≫ P₁₃, which is determined by the closed-loop eigenvalue placement and cannot be eliminated by Q-matrix tuning or LMI optimisation alone, as shown analytically in Section 3.3 and confirmed numerically in Section 5.3.

The proposed e₁-projection resolves this issue with minimal modification to the standard MRAC structure: a single constant vector [P(1,:)B̄_mA; 0; 0] replaces the full P̄B̄ in the adaptation mechanism. The practical loss in theoretical guarantees — the projection departs from the exact Lyapunov descent — is recovered by σ-modification, which closes the proof to UUB.

The early-phase tracking error (t < 5 s, RMS ≈ 6.4 mm) originates from the plant stiction regime. During this phase M̂ grows from 3.97 × 10⁻³ to approximately 0.007 mA/mm, which is insufficient to overcome the static friction threshold of ≈ 108 bar differential pressure. This is a physical limitation of the actuator and not an artefact of the adaptation law; once M̂ crosses a critical value the plant breaks free and tracking improves rapidly. The convergence behaviour could be accelerated by a warm-started M̂(0) derived from an offline friction identification, or by a bang-bang initialisation phase.

The simulation model uses initial hydraulic pressures from experimental data (PA₀ ≈ 71.3 bar), which create a non-zero initial differential pressure PL₀ ≈ −0.69 bar. This initial pressure offset is explicitly accounted for by initialising the reference model integrator from the same state rather than from zero, ensuring e(0) ≈ 0. This detail is important in practice: a zero reference model initialisation in the presence of a large initial PL can cause the adaptation to develop systematic bias in the first few seconds.

Future work will address experimental validation on the physical test bench, including comparison with a fixed-gain pole-placement controller and a PID baseline. Extension to position-dependent friction compensation and online parameter identification of the LuGre model parameters is also of interest.

# 7. Conclusion

This paper has presented a comprehensive MRAC design for a nonlinear EHA system, identifying and resolving a structural limitation of the standard Lyapunov adaptation law that arises specifically in systems with B = [0, 0, b₃]ᵀ. The key contributions are:

- A formal analysis showing that P₃₃/P₁₃ ≈ 771 for the standard Q = I Lyapunov solution, making the pressure error state e₃ dominant in e′PB regardless of operating conditions.

- A position-channel projection φ = e₁·(P₁₃b₃) that eliminates the dominance problem and achieves 0.45 mm RMS steady-state tracking error on the nonlinear model.

- A complete UUB stability proof via Lyapunov analysis with Young's inequality and σ-modification, providing a rigorous theoretical foundation for the projection-based adaptation.

- An LMI-based comparative study using YALMIP/MOSEK demonstrating that P-matrix optimisation alone cannot overcome the structural limitation, confirming that the projection approach is necessary.

The σ = 0.001 modification yields a convergence time of ≈ 5 s and is 6× faster than the unmodified adaptive law while preserving steady-state accuracy. The design framework is directly applicable to other hydraulic and pneumatic actuator systems sharing the same B-matrix structure.

# References

[1] Merritt, H.E. (1967). Hydraulic Control Systems. Wiley.

[2] Jelali, M., & Kroll, A. (2003). Hydraulic Servo-systems: Modelling, Identification and Control. Springer.

[3] Manring, N.D., & Fales, R.C. (2019). Hydraulic Control Systems (2nd ed.). Wiley.

[4] Astrom, K.J., & Wittenmark, B. (2008). Adaptive Control (2nd ed.). Dover.

[5] Slotine, J.J., & Li, W. (1991). Applied Nonlinear Control. Prentice-Hall.

[6] Kocagil, B.M., Ozkan, B., & Mahmutyazicioglu, G. (2022). Model Reference Adaptive Control of an Electrohydraulic Manipulator. Journal of Vibration and Control, 28(13–14), 1811–1828.

[7] Yao, J., Jiao, Z., & Ma, D. (2014). Extended-State-Observer-Based Output Feedback Nonlinear Optimal Control of Hydraulic Systems with Backstepping. IEEE Transactions on Industrial Electronics, 61(11), 6285–6293.

[8] Ioannou, P.A., & Sun, J. (1996). Robust Adaptive Control. Prentice-Hall.

[9] Narendra, K.S., & Annaswamy, A.M. (2005). Stable Adaptive Systems. Dover.

[10] Lofberg, J. (2004). YALMIP: A toolbox for modeling and optimization in MATLAB. Proceedings of the IEEE CACSD Symposium, 284–289.

[11] Canudas de Wit, C., Olsson, H., Astrom, K.J., & Lischinsky, P. (1995). A New Model for Control of Systems with Friction. IEEE Transactions on Automatic Control, 40(3), 419–425.

[12] Won, D., Kim, W., Shin, D., & Chung, C.C. (2015). High-Gain Disturbance Observer-Based Backstepping Control with Output Tracking Error Constraint for Electro-Hydraulic Systems. IEEE Transactions on Control Systems Technology, 23(2), 787–795.